The Computer Communication Lab (236340)

Winter 2009-2010

OLSRv2 protocol simulation and analysis

### Final Report

# Submitted by:

###### Assaf Israel

###### Eli Nazarov

###### Asi Bross

### Table of contents:

1	Introduction	3

1.1	OLSR introduction		3

1.2	Neighborhood Discovery Protocol (NHDP)		3

1.3	Optimized Link State Routing Ver. 2 (OLSRv2)		4

2	Project Description	6

2.1	Purpose		6

2.2	Main Goals		6

2.2.1	 Utilization Analysis		6

2.2.2	 Network Reliability		6

2.2.3	 Topology		6

2.2.4	 Scalability		6

2.3	Stretched Goals		7

2.3.1	 Topology mobility		7

2.3.2	 Secondary MPR sets		7

2.4	Output		7

3	General Layout	8

4	Main Modules		8

4.1	GUI	9

4.2	Log	9

4.3	Tasks Queue		10

4.4	Event Generator		10

4.5	Dispatcher		10

4.6	Topology Manager		11

4.7	OLSRv2 Layer		11

5	Main Algorithm		12

5.1	Event Generation		12

5.2	Protocol implementation		12

5.3	Dispatcher		14

5.4	Topology Manager		14

6	Appendixes		16

6.1	Appendix 1: Estimated project timetable		16

6.2	Appendix 2: Requirements and Assumptions		16

6.2.1	 Assumptions		16

6.2.2	 Requirements		16

# Introduction

## OLSR introduction

The Optimized Link State Routing Protocol (OLSR) is developed for mobile ad hoc networks.  It operates as a proactive protocol (table driven) which exchanges topology information with other nodes of the network regularly.  It was developed to work independently from other protocols.  Likewise it makes no assumptions about the underlying link-layer.  The protocol inherits the stability of a link state algorithm and has the advantage of having routes immediately available when needed due to its proactive nature.  OLSR stack location is beneath the transport layer as supplement to the network layer.  Each node can operate as a router to different implementations of local networks.

OLSR is well suited to larger and denser mobile networks then other naive routing options.  The larger and more dense a network, the more optimization can be achieved.  OLSR uses hop-by-hop routing, i.e., each node uses its local information to route packets.

OLSR is designed to work in a completely distributed manner and does not depend on any central entity.  The protocol does not require reliable transmission of control messages since each node sends control messages periodically, and can therefore sustain a reasonable loss of some messages.

## Neighborhood Discovery Protocol (NHDP)

The NHDP objective is to maintain a 1-hop and 2-hop neighbors set for each node in a Mobile Ad-hoc Network (MANET). This enables the node, to use this information later, when implementing the OLSR protocol. NHDP is a table driven protocol, meaning maintenance messages are used in order to maintain the nodes databases.

NHDP uses only one type of messages, called "Hello" messages. These messages are sent between each node and every node that is in its broadcast radius (1-hop neighbors), but not beyond. Meaning, these messages are not forwarded by any receiver. Their role is passing information about a node's 1-hop neighbors to its 1-hop neighbors. A node, which receives a message, can add the sender to its 1-hop neighbors set. If the receiver is stated in the senders 1-hop neighbors set, he can state that the link is bi-directional (i.e. messages are send and received by both parties). The receiver construct a 2-hop neighbors set by joining all the received sender's 1-hop neighbors sets and subtracting its own 1-hop neighbors set. Meaning, node C is a 2-hop neighbor of node A, if it's a 1-hop neighbor of node B that is a 1-hop neighbor of node A, and C is not a 1-hop neighbor of node A.

Figure ‎1-1

"Hello" messages are sent by every participating node, and by every node's network interface, in a manner that will extend the network connectivity as much as possible. E.g. – node C can be a 2-hop neighbor of node A, via node B but not share the same network interface with node B that B shares with A. This behavior allows the protocol to build the most comprehensive and flexible 1-hop & 2-hop topology map, so future data messages could "switch" network interfaces in order to achieve the best and fastest possible connectivity to their destination.

"Hello" messages are sent periodically according to an adjustable Refresh Interval. This means that if a message from a 1-hop neighbor is not received by a node, then this connection is considered lost, and the neighbor is removed from the 1-hop neighbors set. At the next "Hello" message that will be sent from the nodes that noticed their neighbor's disappearance, the change will be noted so other neighbors could be aware of the change as quickly as possible.

A node does not have to wait until the next scheduled "Hello" message transmission, in order to alert the change (new/lost/changed status link). He can issue a dynamically scheduled "Hello" message after a certain Minimum Interval from the last message transmission.

## Optimized Link State Routing Ver. 2 (OLSRv2)

Optimized Link State Routing Ver.2 (OLSRv2) is a proactive protocol (table driven) that uses NHDP protocol described in section 1.2. OLSRv2 is optimized routing protocol for MANET networks that can handle dynamic topology.

The main concept of the protocol is the use of Multipoint Relays (MPRs). MPRs are used in the OLSR protocol to minimize the overhead of flooding messages in the network by reducing redundant retransmissions in the same region.  Each node in the network selects a set of nodes in its symmetric 1-hop neighborhood (see ‎1.2) that may retransmit its messages.  This set of selected neighbor nodes is called the "Multipoint Relay" (MPR) set of that node.  The neighbors of node X which are not in its MPR set, receive and process broadcast messages but do not retransmit broadcast messages received from node X.  In route calculation, the MPRs are used to form the route from a given node to any destination in the network.

The set of MPRs should cover all the 2-hop (see ‎1.2) neighbors of the node, meaning that the union of all neighbors of MPRs gives the group of all nodes until the 2-hop degree of the MPRs selector.

Basically, the route between two nodes in the network is a sequence of hops through MPRs. The last MPR is the target node or, the target is a 1-hop neighbor of the last MPR.

A node’s request from another to be an MPR for it, is passed via attaching new information to “Hello” messages generated by the underlying NHDP.

OLSRv2 defines only one type of message, Topology Control (TC).

“Hello” messages are being modified by OLSRv2 to contain the selected MPRs set of the source node.  Upon receiving “Hello” messages, if the target node was selected as MPR, it will update its status to be an MPR for the source node. “Hello” messages are never forwarded by the OLSRv2 protocol.

“TC” messages include the set of all nodes that selected the source node as a MPR. Also, this message may contain relevant information regarding the different interfaces of the node and the network (if such exist) that are connected to this node. “TC” messages are always forwarded, unless this message was already sent by this station, and are broadcasted to all nodes in the network.

OLSRv2 keeps different information tables, Topology Information Base, that hold information regarding the MPR selectors of this node, network topology information and information regarding routes to all nodes in the network.

Furthermore, OLSRv2 keeps information regarding all “TC” messages that this station receives processes or forwards – Processing and Forwarding Information Base. This information helps to lower the amount of “TC” messages that are being sent by each node.

“TC” messages are being generated and transmitted periodically by the node once in a specific time, or due to some changes in the topology of the node. Topology change is basically a change in contents of Topology Information Base.

When a packet needs to be transmitted from node A to node B, node A will send it to the first MPR in the current route known to node A. Each MPR will forward the packet to the next MPR in the route.

Figure ‎1-2

# Project Description

## Purpose

The main purpose of this project was to create a simulator for the OLSRv2 protocol.  Investigate and analyze the behavior and performance of the protocol with different characteristics of the network.  The simulator was created with a wide range of adjustable parameters that can produce a wide spectrum of analytic measurements.

## Main Goals

The simulator that we created can produce a wide range of analytic measurement; hence we decided to focus our research on the following topics.

### MPR Selection Analysis

MPR selection can have a significant impact on the network utilization, a minimal selection of 1-hop MPRs that cover the entire 2-hop neighborhood of a single node may result in a better network utilization in comparison to a selection of the entire 1-hop neighborhood as MPRs.

#### Minimal 1-Hop Set

#### All 1-Hop Set

#### Results

### Data Send Mode Analysis

We decided to compare between the propagation of data packets through the MPRs vs. the propagation of data packets through the entire 1-hop neighborhood.

#### MPRs only

#### All neighbors

#### Results

### Topology Analysis

The simulator we created supports setting the topology layout in one of two modes, either UNIFORM or CLUSTER.  We decided to investigate the throughput received in each of these topology layouts.

#### Uniform

#### Cluster

## Output

### GUI

The output generated by the GUI shows the state of the stations and the data packets transmissions while they occur, the size attributes of the simulated world can be altered through the "World size" entry in the Layout tab.  The speed in which the graphical simulation is presented can be changed during the simulation run by altering the "Simulation speed" entry in the Simulation tab.

### SQL

One of the goals of this simulator is the ability to analyze the protocol under different settings of protocol parameters, topology settings, different speed of station movement etc. In order to accomplish this we have implemented a logger (see ‎4.3) that is purged at the end of the simulation in to SQL database, in addition to the excel file. The implementation is done using MySQL plug-in for eclipse.

#### Log Structure

The log consists of the following columns.

INDX - The index of the raw in the log.

VIRTUAL_TIME -  The simulation time.

NODE_ID -  The ID of station that made the log

EVENT_TYPE -  The type of the event.

GLOBAL_SOURCE -  The initiator source of the message (e.g. the source of the Data Message).

LOCAL_SOURCE -  The id of the station from which the message was received.

LOCAL_TARGET - The station id that received the message

GLOBAL_TARGET - The destination of the message

X_COOR, Y_COOR - The coordinates of the station in the plane.

RADIUS - The transmission radius of the station

ERROR - Boolean flag stating if some error accrued.

DETAILS  - More details regarding this log

#### Event Types

Here are the main EVENT_TYPE values.

DATA_SENT_EVENT – Process of data message.

BUSSY_MSG_IGNORED – The station is busy, thus ignored the received message.

DATA_SENT_FROM_SOURCE – Message was sent from the source.

DATA_REACHED_2_TARGET – Message reached its destination.

DATA_NOT_SENT_NO_ROUTE – Message was not sent because the source didn’t find route

DATA_LOSS_NO_ROUTE – Relay station didn’t found route, message was lost.

DATA_IGNORED_AT_RELAY – Station received message that was not addressed to her.

DATA_SENT_FROM_RELAY – Relay station sent message to the next station in the route.

DATA_REACHED_2_RELAY – Relay station on the route received data message.

LOCAL_TARGET_NOT_PHYSIBLE – There was a send to the station that doesn’t exist physically (station didn’t learned yet that the station is no longer there).

TTL_LIMIT_REACHED – TTL of the data message is over (the message is dropped).

# Topology Layout

The OLSRv2 illustrates a MANET network of nodes that move dynamically. The simulator simulates both high and low dense areas. We have implemented variations for the topology of the stations. Uniform topology is a topology where the stations are spread uniformly over the plane. The Cluster topology is when the stations are spread over the plane in groups. Using this topology we can check when the messages are routed through only few nodes in order to reach stations in other clusters (“bottle-neck” stations).

Figure 3-1: Uniform formation

Figure 3-2: Cluster Distribition

# Implementation

## Class Diagram

## GUI [Asi]

The GUI allows the user to set different specification for the simulator execution it enables the user to see the current state of the MANET by painting a graphical presentation of the nodes and the transmission of data messages.

### Input Parameters

#### Simulation parameters

- Simulation end time – The amount of logical time units the simulation execution should run.

- Maximum stations – The maximum number of stations that can exist simultaneously.  In static mode this represent the number of stations (No topology changes occur), in dynamic mode it represent the initial and maximum number of stations.

- Reception radius – Represents the radius of reception of each station.

- Topology poissonic rate – The expected number of topology events that can occur in a single time unit.

- Data events poissonic rate - The expected number of data messages events that can occur in a single time unit.

- Simulation speed – The current speed of the simulation graphic display, it sets the rate at which a single simulation tick occurs, this parameter can be changed during the simulation run, since this only effect the GUI representation it does not have any effect on the simulation results.  The values for this parameter are SLOW, NORMAL, FAST, REAL TIME – the "REAL TIME" value displays the current simulation state while the others may have a certain delay in the graphic presentation.

- Stations speed – In dynamic mode it set the amount of time the station moves between two points.

#### Layout parameters

- World size – Sets the size of the area in which stations can exist, the input value is in the format of <width>x<height>, in the GUI representation it is changed dynamically and covers a pixel area in the given size, the boundaries are between 100x100 and 500x500.

- Stations mode – Set the nature of the stations, STATIC value means that all the stations are created in the beginning of the simulations and they don’t change their position, DYNAMIC value means that topology events can occur which can cause new stations to appear, existing stations can be destroyed and existing stations can change their position.

- Layout mode – Set the topology layout of the stations, the values are UNIFORM and CLUSTER, CLUSTER means that the stations are organized in groups.

- Cluster radius – The radius of a single cluster, all the stations are inside this radius.  Applicable only if layout mode is CLUSTER.

- Number of clusters – The total number of clusters in the topology.  Applicable only if layout mode is CLUSTER.

#### Protocol parameters

- Entry validity period – The amount of unit time a table entry is valid in the station's control tables.

- Hello message interval – The amount of unit time between consecutive hello message transmissions of the same station.

- TC message interval - The amount of unit time between consecutive TC message transmissions of the same station.

- Transmission time – The amount of unit time it takes for a transmission of a packet.

- Protocol mode – The mode in which control packets are being transferred, NORMAL mean through all 1-hop neighbors, ALL MPRs mean through all the MPRs of a station.

- Protocol data send mode – The mode in which data packets are being transferred, NORMAL means through any 1-hop neighbor, MPRs mean through MPRs only.

### World Topology

The stations location are represented as circles on a <width>x<height> panel that represents the world in which the stations reside.  The stations distribution can be defined through the Layout mode entry which enables a uniform or cluster distribution.  Data packets that are sent between stations are represented as a line that connects the two participating stations and is visible for a period of "Transmission time" as specified by the user.

## Log

This module is responsible for all the data handling of the system. It acts as a hub for different components to log their data in.

Since data from all parts of the system concentrate here, the Log is the idle place to process the data and produce more meaningful numbers, like the different throughput calculations, and data messages failure rate.

Beside the regular data reports the Log will also hold Error reports, that contain information about system health, and also reports about data messages that have failed to reach their destination (points to protocol failure).

The main type of data the Log will collect is information about different messages that will spark from the different OLSRv2 Layers (Nodes). Each control message ("Hello" or "TC" message) as well as each data message that is being sent (passed to the Tasks Queue) will be logged for later processing.

## Event Generator

The Event Generator is responsible for the creation of two types of events:

- Topology events – Creation/Migration/Destruction of stations.

- Data messages – These events simulate data, that’s being received by the OLSR Layer at a certain node, and is designated to another node in the network.

In order to safely create these events, the Event Generator needs to hold a set of the nodes labels and coordination.  It will query the set prior to any event generated in order to maintain consistency.

For example, upon creating a new station the Event Generator should check that a station does not already exists, with the same parameters (id, location). Another example is when creating Data event, the Event Generator should check if the source or target stations exists.

For more information about the specific algorithms that are used to handle these events, see section ‎5.1.

## Dispatcher

The Dispatcher’s name basically reveals its role. It’s designed to retrieve the top task from the Tasks Queue (The next task in a chronological order), inspect it, and redirect it to the relevant objects which in turn process it.

The Dispatcher specific behavior depends on the type of task it has retrieved from the Tasks Queue:

- Topology events – are passed directly to the Topology Manager, who in turn updates its records based on the information given.

- Data messages – are passed to the OLSRv2 Layers which identifies as the source nodes of the messages.

- “Hello” or “TC” messages – are passed to all of the OLSRv2 Layers that represents nodes which are within the Reception Radius of the originator node location. The list of nodes which answer this criterion is given to the Dispatcher by requesting the Topology Manager which has the knowledge regarding the nodes physical whereabouts in space.

Whenever the Dispatcher notices that an event destination is missing (e.g. Data message is sent to a non-existing node) it will log this event at the Log and will discard the event.

## Topology Manager

The Topology Manager Job is to maintain information about the simulated physical attributes of nodes. The combined data of all the nodes attributes defines the network physical picture at a certain time. The attributes that will be stored per node within the Topology Manager are:

- Coordinates in space – the simulated physical location at the defined 2D space. These Coordinates can be changed by the Topology Manager upon receiving a Topology event from the Dispatcher which instructs it to migrate a certain node to a new location (considering that the new location is still within the defined space boundaries)

- Reception Radius – each node might have a different reception radius, simulating changes that might be induced because of different platforms or terrain conditions that real world station are expose to.

The Topology Manager may be asked by the Dispatcher to supply a list of nodes which Coordinates are within a certain node’s Reception Radius. That way the Dispatcher can decide which OLSRv2 Layers needs to be on the receiver end of a certain “Hello” or “TC” message task.

## Protocol [ELI]

This module implements the OLSRv2 logic of a single node it a MANET environment. This module is split into two Three-layers:

- Protocol Layer

- NHDP Layer

- OLSR Layer

### Protocol Layer

This is a logical layer that makes the connection between the different parts of the simulator and the protocol layers. Due to the fact that the simulator is event driven all messages are being sent as events to the dispatcher and all the intervals (Hello/TC) are measured by special events,  thus when the dispatcher processes an event it invokes an appropriate method in the protocol layer of the station. This method translates the event received from the dispatcher to an appropriate message of the protocol or an event related to the work flow, e.g. Hello/TC interval ended, and passes it for process in the protocol layers. Also this layer is responsible for generating, processing and routing Data messages according to different topology data sets that are maintained by the OLSRv2 and NHDP layers.

### NHDP Layer

Implements the NHDP layer. NHDP layer is responsible for processing and generating Hello messages. Upon receiving Hello messages this layer updates the different data sets it maintains and generates Hello messages if any change happened in the 1-hop topology of the station.

### OLSRv2 Layer

Implements the OLSRv2 logic. This layer is responsible for adding additional information to the generated Hello messages (MPR information) as detailed in the OLSR description (Section ‎1.3), as well as generating and processing “TC” messages and Data messages. Upon receiving a TC message the layer updates different data sets that hold information regarding the global topology of the network, re-calculates MPRs for this station and calculates routes to all the stations it learned.

# Appendixes

## Requirements and Assumptions

### Assumptions

- Low level implementation details as described in draft- ietf-manet-olsrv2-10 and in draft-ietf-manet-nhdp-10 like packet formation or jitter aspects are not in the scope of this simulation. Requirements

- The simulator is implemented in Java, so JVM is needed for execution.

- MySQL should be installed in order to get SQL support.

## Future Work